Here is the comprehensive GitHub report material template for Experiment 4, focusing on Event Flags (Event Groups) and Software Timers, structured to match the academic standards and format of the previous experiments.

# Experiment 4: Advanced Synchronization using Event Flags and Software Timers on STM32F446RE

## 1. Objective

- To understand advanced synchronization mechanisms using Event Flags (Event Groups) to coordinate tasks based on multiple combined conditions.

- To learn how to implement and utilize Software Timers for executing periodic or one-shot timed callbacks without relying on dedicated hardware timers.

- To configure and initialize these FreeRTOS primitives on the STM32F446RE Nucleo board using the CMSIS-RTOS V1 API layer.

## 2. Hardware and Software Requirements

- Hardware:

  - STM32F446RE Nucleo-64 Development Board.

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY) to observe system logs and timing events.

## 3. Theoretical Background

### Event Flags (Event Groups)

While Semaphores are excellent for simple signaling between two tasks, they fall short when a task needs to wait for multiple events to occur, or when multiple tasks need to be notified of the same event. Event Flags resolve this by allowing a set of binary status flags (bits) to be grouped together.

A task can block waiting for:

- AND Dependency: All specified bits to be set (e.g., Wait until Sensor_A_Ready AND Sensor_B_Ready are both true).

- OR Dependency: Any one of the specified bits to be set (e.g., Wait until Error_Flag OR User_Button_Pressed becomes true).

Once the condition is met, the waiting task automatically wakes up, with the option to clear the bits upon exiting.

### Software Timers

Software Timers allow a developer to execute a specific function (a callback function) at a set time in the future, or repeatedly at regular periodic intervals.

- They are managed entirely by the FreeRTOS kernel via a hidden system daemon task (the Timer Service Task), meaning they do not consume hardware timer peripherals (like TIM2 or TIM3).

- Types of Software Timers:

  - One-Shot Timers: Run exactly once when started, execute their callback function, and then enter a dormant state (e.g., turning off an LED after a specific duration).

  - Auto-Reload Timers: Automatically restart themselves every time they expire, creating a continuous periodic execution cycle (e.g., polling a battery level every 5 seconds).

## 4. Architectural Configuration in STM32CubeMX

To enable these advanced features under FreeRTOS:

- Timer Service Configuration: Under Middleware > FREERTOS > Config parameters, ensure USE_TIMERS is set to Enabled. Set the TIMER_TASK_PRIORITY highly enough to handle callbacks instantly.

- Event Group Allocation: Under the FreeRTOS middleware layout, ensure event flag capabilities are activated (often configured programmatically in code using CMSIS-RTOS V1, as CubeMX's GUI native support varies between CMSIS versions).

- Software Timers tab: Navigate to the Timers tab, click Add, define your timer type (osTimerPeriodic or osTimerOnce), and bind it to a custom C callback function.

## 5. Firmware Implementation Structure

The following code pattern demonstrates an Event-Driven Controller. Two separate conditioning routines set specific status flag bits. A master controller task blocks until both bits are set simultaneously. Additionally, a periodic Software Timer runs independently to pulse a system heartbeat log.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Private variables ---------------------------------------------------------*/

// Task Handles

osThreadId ControllerTaskHandle;

osThreadId InputTaskHandle;

// Software Timer Handle

osTimerId HeartbeatTimerHandle;

// Event Group / Event Flags Definitions

// In CMSIS-RTOS V1, we can leverage signals bound directly to tasks, or define event groups.

#define BIT_0_SENSOR_READY  (1 << 0)

#define BIT_1_SYSTEM_OK     (1 << 1)

/* UART handle for console logs */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartControllerTask(void const * argument);

void StartInputTask(void const * argument);

void HeartbeatCallback(void const * argument);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

/* --- Create the Software Timer --- */

// Defines a periodic timer linked to the HeartbeatCallback function

osTimerDef(HeartbeatTimer, HeartbeatCallback);

HeartbeatTimerHandle = osTimerCreate(osTimer(HeartbeatTimer), osTimerPeriodic, NULL);

/* --- Start the Software Timer --- */

// Starts the timer with a periodic interval of 1000ms

osTimerStart(HeartbeatTimerHandle, 1000);

/* --- Create Operating Threads --- */

/* Controller Task (High Priority - Waits for event signals) */

osThreadDef(ControllerTask, StartControllerTask, osPriorityAboveNormal, 0, 128);

ControllerTaskHandle = osThreadCreate(osThread(ControllerTask), NULL);

/* Input Task (Normal Priority - Distributes event signals) */

osThreadDef(InputTask, StartInputTask, osPriorityNormal, 0, 128);

InputTaskHandle = osThreadCreate(osThread(InputTask), NULL);

/* Start RTOS Scheduler */

osKernelStart();

while (1)

{

// Kernel controlled loop

}

}

/*------------------------- Callback and Thread Implementations --------------*/

/**

* @brief  Software Timer Callback function. Runs periodically in the context

* of the RTOS Timer Daemon Task. (Keep processing short, do not block here!)

*/

void HeartbeatCallback(void const * argument)

{

char msg[] = "[SOFTWARE TIMER] Heartbeat active... System healthy.\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg, sizeof(msg)-1, 100);

}

/**

* @brief  Input Task: Simulates external events occurring over time by setting flags.

*/

void StartInputTask(void const * argument)

{

for(;;)

{

osDelay(2000); // Simulate processing time

char msg1[] = "Input Task: Sensor Ready Bit Set.\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg1, sizeof(msg1)-1, 100);

// Send event signal (Bit 0) to the Controller Task

osSignalSet(ControllerTaskHandle, BIT_0_SENSOR_READY);

osDelay(2000); // Simulate subsequent processing delay

char msg2[] = "Input Task: System Integrity Check Bit Set.\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg2, sizeof(msg2)-1, 100);

// Send event signal (Bit 1) to the Controller Task

osSignalSet(ControllerTaskHandle, BIT_1_SYSTEM_OK);

}

}

/**

* @brief  Controller Task: Blocks until BOTH conditions are fully satisfied.

*/

void StartControllerTask(void const * argument)

{

osEvent evt;

for(;;)

{

char msg_wait[] = "--> Controller Task: Blocked, waiting for ALL conditions...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_wait, sizeof(msg_wait)-1, 100);

// Wait for BOTH BIT_0 and BIT_1 simultaneously. (0xFFFF filters target signals)

// osWaitForever places the task into an efficient non-loading state.

evt = osSignalWait(BIT_0_SENSOR_READY | BIT_1_SYSTEM_OK, osWaitForever);

if (evt.status == osEventSignal)

{

// Execution resumes only when conditions are satisfied

char msg_exec[] = "\r\n[SYSTEM EVENT EXECUTION] Both conditions satisfied! Processing Action...\r\n\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_exec, sizeof(msg_exec)-1, 100);

}

}

}

## 6. Procedural Steps for Lab Execution

- Initialize Project Properties: Access your ongoing FreeRTOS configuration template inside STM32CubeIDE.

- Configure Software Timer Core: Click into your .ioc configuration canvas. Navigate to Middleware > FREERTOS > Config parameters. Verify that USE_TIMERS is switched to Enabled.

- Set Up Software Timer Handle: Move to the Timers tab under FreeRTOS configurations, click Add, define an identification handle name, choose osTimerPeriodic, and link it to your custom Callback function handler.

- Task Allocations: Define ControllerTask and InputTask within the Tasks and Queues tab.

- Code Generation: Press Ctrl + S to save configurations and trigger code generation.

- Program the App Logic: Open main.c. Write the osTimerStart() implementation right before the scheduler initialization block. Then, fill in your logic using osSignalSet() and osSignalWait() to drive synchronous step executions.

- Verify & Execute: Build your project binaries, clear any syntax warnings, upload the compiled script to your target STM32 Nucleo hardware, and connect your serial visual terminal.

## 7. Observations & Verification

- Software Timer Consistency: The serial terminal cleanly tracks the [SOFTWARE TIMER] trace message repeating exactly every 1000 milliseconds, entirely unbothered by what the other tasks are doing.

- Conditional Synchronization: The Controller Task stays in a completely non-blocking sleep state even after the first bit (BIT_0_SENSOR_READY) is flagged. It only awakens to print its execution notification the exact millisecond BIT_1_SYSTEM_OK is fulfilled by the companion task, confirming successful AND condition synchronization.