Here is the comprehensive GitHub report material template for Experiment 9, focusing on Deadlocks and Resource Management, structured to match the academic formatting of the previous experiments.

# Experiment 9: Resource Deadlocks and Concurrency Anomalies in FreeRTOS on STM32F446RE

## 1. Objective

- To simulate, observe, and analyze the classic concurrency problem of a Deadlock in a real-time system.

- To understand the four necessary conditions (Coffman conditions) that allow a deadlock to occur.

- To implement and verify robust architectural solutions—specifically Resource Ordering and Timeouts—on the STM32F446RE Nucleo board using the CMSIS-RTOS V1 API layer.

## 2. Hardware and Software Requirements

- Hardware:

  - STM32F446RE Nucleo-64 Development Board.

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY).

## 3. Theoretical Background

### What is a Deadlock?

A Deadlock is a catastrophic concurrency failure where two or more tasks are permanently blocked because each task is waiting for a resource that is held by the other. Because both tasks are blocked waiting for a token release, neither task ever executes the code needed to release the lock, freezing that part of the application indefinitely.

### The Classic "Deadlock Embrace" Scenario

Consider two tasks (Task A and Task B) and two shared resources protected by Mutexes (Mutex 1 and Mutex 2):

- Task A runs, successfully takes Mutex 1, and gets preempted by the scheduler.

- Task B runs, successfully takes Mutex 2, and then attempts to take Mutex 1. Since Mutex 1 is held by Task A, Task B enters the Blocked state.

- Task A resumes execution and attempts to take Mutex 2. Since Mutex 2 is held by Task B, Task A also enters the Blocked state.

Both tasks are now locked in a mutual embrace, waiting for each other indefinitely.

### The Coffman Conditions

For a deadlock to occur, four conditions must simultaneously hold true:

- Mutual Exclusion: Only one task can hold a resource at any given time.

- Hold and Wait: A task must hold a resource while actively waiting to acquire another.

- No Preemption: Resources cannot be forcibly stripped away from a task holding them.

- Circular Wait: A closed chain of tasks exists where each task holds a resource needed by the next task in the chain.

### Prevention and Resolution Techniques

- Resource Ordering (Prevention): Forces all tasks in the system to always acquire shared resources in the exact same strict sequential order (e.g., always acquire Mutex 1 before Mutex 2). This effectively breaks the Circular Wait condition.

- Mutex Timeouts (Resolution): Instead of waiting indefinitely (osWaitForever), tasks request a resource with a defined time boundary (e.g., 100 ms). If the lock cannot be acquired within that window, the call fails gracefully, allowing the task to back out, release its own held locks, and retry later.

## 4. Architectural Configuration in STM32CubeMX

To configure your project workspace to safely analyze deadlocks:

- Timebase Source: Ensure System Core > SYS > Timebase Source is set to a dedicated peripheral timer (e.g., TIM1).

- Middleware Integration: Under FREERTOS, navigate to the Mutexes tab.

- Initialize Resource Tokens: Click Add to create two distinct resource handles:

  - Resource_A_MutexHandle

  - Resource_B_MutexHandle

- Task Definitions: Under the Tasks tab, register two independent tasks (TaskA_Thread and TaskB_Thread) with identical or near-identical priority allocations to easily capture preemption steps.

## 5. Firmware Implementation Structure

The following structural template demonstrates how to induce a deadlock and how to resolve it using a Resource Timeout strategy controlled by an evaluation macro switch.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Configuration Macro -------------------------------------------------------*/

// Set to 1 to simulate a Deadlock. Set to 0 to implement Timeout Resolution.

#define INDUCE_DEADLOCK_MODE   1

/* Private variables ---------------------------------------------------------*/

// Task Handles

osThreadId TaskA_Handle;

osThreadId TaskB_Handle;

// Resource Protection Locks

osMutexId Resource_A_MutexHandle;

osMutexId Resource_B_MutexHandle;

/* UART handle for fault monitoring */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartTaskA(void const * argument);

void StartTaskB(void const * argument);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

char msg_init[] = "--- Concurrency & Deadlock Detection Lab Booted ---\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_init, sizeof(msg_init)-1, 100);

/* --- Allocate Mutex Pools --- */

osMutexDef(Resource_A_Mtx);

Resource_A_MutexHandle = osMutexCreate(osMutex(Resource_A_Mtx));

osMutexDef(Resource_B_Mtx);

Resource_B_MutexHandle = osMutexCreate(osMutex(Resource_B_Mtx));

/* --- Allocate Threads --- */

osThreadDef(TaskA, StartTaskA, osPriorityNormal, 0, 128);

TaskA_Handle = osThreadCreate(osThread(TaskA), NULL);

osThreadDef(TaskB, StartTaskB, osPriorityNormal, 0, 128);

TaskB_Handle = osThreadCreate(osThread(TaskB), NULL);

/* Start RTOS Scheduler */

osKernelStart();

while (1)

{

// Kernel controlled loop

}

}

/*------------------------- Task Implementations -----------------------------*/

/**

* @brief  Task A: Acquires Resource A, then tries to acquire Resource B.

*/

void StartTaskA(void const * argument)

{

for(;;)

{

char msg_a1[] = "Task A: Requesting Resource A...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_a1, sizeof(msg_a1)-1, 100);

if (osMutexWait(Resource_A_MutexHandle, osWaitForever) == osOK)

{

char msg_a2[] = "[SUCCESS] Task A secured Resource A. Simulating work...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_a2, sizeof(msg_a2)-1, 100);

// Artificial delay forces a context switch, allowing Task B to pre-empt

osDelay(50);

char msg_a3[] = "Task A: Now requesting Resource B...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_a3, sizeof(msg_a3)-1, 100);

#if INDUCE_DEADLOCK_MODE

// Dangerous Call: Wait forever. Causes infinite freeze if Resource B is held.

if (osMutexWait(Resource_B_MutexHandle, osWaitForever) == osOK)

{

osMutexRelease(Resource_B_MutexHandle);

}

#else

// Safe Alternative: Timeout resolution. Only wait 200ms before backing out.

if (osMutexWait(Resource_B_MutexHandle, 200) == osOK)

{

char msg_ok[] = "[SUCCESS] Task A completed full routine.\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_ok, sizeof(msg_ok)-1, 100);

osMutexRelease(Resource_B_MutexHandle);

}

else

{

char msg_fail[] = "!!! [TIMEOUT WATCHDOG] Task A failed to secure Resource B. Backing out... !!!\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_fail, sizeof(msg_fail)-1, 100);

}

#endif

osMutexRelease(Resource_A_MutexHandle);

}

osDelay(500);

}

}

/**

* @brief  Task B: Acquires Resource B, then tries to acquire Resource A.

* In Deadlock mode, this creates an inversion pattern against Task A.

*/

void StartTaskB(void const * argument)

{

for(;;)

{

char msg_b1[] = "\t\tTask B: Requesting Resource B...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_b1, sizeof(msg_b1)-1, 100);

if (osMutexWait(Resource_B_MutexHandle, osWaitForever) == osOK)

{

char msg_b2[] = "\t\t[SUCCESS] Task B secured Resource B. Simulating work...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_b2, sizeof(msg_b2)-1, 100);

// Give Task A a chance to run and grab its primary lock

osDelay(50);

char msg_b3[] = "\t\tTask B: Now requesting Resource A...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_b3, sizeof(msg_b3)-1, 100);

#if INDUCE_DEADLOCK_MODE

// Dangerous cross-dependency check

if (osMutexWait(Resource_A_MutexHandle, osWaitForever) == osOK)

{

osMutexRelease(Resource_A_MutexHandle);

}

#else

// Safe timeout management

if (osMutexWait(Resource_A_MutexHandle, 200) == osOK)

{

osMutexRelease(Resource_A_MutexHandle);

}

else

{

char msg_fail[] = "\t\t!!! [TIMEOUT WATCHDOG] Task B failed to secure Resource A. Backing out... !!!\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_fail, sizeof(msg_fail)-1, 100);

}

#endif

osMutexRelease(Resource_B_MutexHandle);

}

osDelay(500);

}

}

## 6. Procedural Steps for Lab Execution

- Initialize Project Properties: Open your ongoing FreeRTOS template inside STM32CubeIDE.

- Configure Multi-Mutex Handles: Double-click the project .ioc device map. Navigate to Middleware > FREERTOS > Mutexes and click Add twice to declare Resource_A_Mutex and Resource_B_Mutex.

- Register Threads: In the Tasks section, create TaskA and TaskB with identical priority setups (osPriorityNormal).

- Generate Project Layout: Save your changes (Ctrl + S) to trigger the automated code generation process.

- Program the Deadlock Trap: Open main.c. Set the INDUCE_DEADLOCK_MODE macro configuration flag to 1. Complete the cross-locking thread functions exactly as shown in Section 5.

- Compile and Execute (Test 1): Flash the compiled code onto your Nucleo board. Open your serial console monitor and observe the terminal output when the system locks up.

- Apply Timeout Recovery (Test 2): Modify the configuration macro switch INDUCE_DEADLOCK_MODE to 0. Recompile, reload the code onto the microcontroller, and verify that the system can now gracefully recover from a deadlock lockup.

## 7. Observations & Verification

### Case 1: Deadlock Mode Active (INDUCE_DEADLOCK_MODE 1)

Upon resetting the board, the serial monitor prints the following sequence and then freezes completely:

Plaintext

Task A: Requesting Resource A...

[SUCCESS] Task A secured Resource A. Simulating work...

Task B: Requesting Resource B...

[SUCCESS] Task B secured Resource B. Simulating work...

Task A: Now requesting Resource B...

Task B: Now requesting Resource A...

- Observation: The console output stops completely after these lines. Both tasks are permanently stuck in the Blocked state, waiting for a resource token that will never be released. This verifies a textbook resource deadlock.

### Case 2: Timeout Mitigation Active (INDUCE_DEADLOCK_MODE 0)

When the macro is switched to 0 and a bounded timeout is applied to the mutex requests, the terminal log shows a dynamic, self-healing runtime sequence:

Plaintext

Task A: Requesting Resource A...

[SUCCESS] Task A secured Resource A. Simulating work...

Task B: Requesting Resource B...

[SUCCESS] Task B secured Resource B. Simulating work...

Task A: Now requesting Resource B...

Task B: Now requesting Resource A...

!!! [TIMEOUT WATCHDOG] Task A failed to secure Resource B. Backing out... !!!

!!! [TIMEOUT WATCHDOG] Task B failed to secure Resource A. Backing out... !!!

Task A: Requesting Resource A...

[SUCCESS] Task A secured Resource A. Simulating work...

[SUCCESS] Task A completed full routine.

- Observation: Instead of locking up forever, the tasks detect that the resource is unavailable after waiting for 200 ms. They drop out of their conditional blocks, release their primary locks, and yield. This breaks the circular wait condition and allows the scheduler to resolve the conflict, proving that timeout bounds are an effective way to keep critical real-time embedded systems stable and resilient.