# Experiment 1: Introduction to RTOS and Task Creation on STM32F446RE

## 1. Objective

- To understand the core fundamentals of a Real-Time Operating System (RTOS) and how it differs from a conventional bare-metal (polling/interrupt) system.

- To configure and initialize FreeRTOS on the STM32F446RE Nucleo board using the STM32CubeIDE environment via the CMSIS-RTOS V1 API layer.

- To successfully create multiple concurrent tasks with distinct priority levels and observe deterministic multitasking behavior.

## 2. Hardware and Software Requirements

- Hardware: * STM32F446RE Nucleo-64 Development Board (featuring an ARM Cortex-M4 core running up to 180 MHz, 512 KB Flash, 128 KB SRAM).

  - Mini-B USB Cable (for programming and debugging via the onboard ST-LINK/V2-1).

- Software:

  - STM32CubeIDE (Integrated Development Environment for C/C++ compilation and hardware configuration).

  - Serial Monitor Client (e.g., TeraTerm or Putty) for debugging outputs via Virtual COM Port (USART2).

## 3. Theoretical Background

### Real-Time Operating System (RTOS) Overview

Unlike standard operating systems (like Windows or Linux) optimized for high throughput, an RTOS is designed to execute applications with exact, predictable timing constraints. The key feature of an RTOS is determinism, meaning that the operational response time to an event can be guaranteed within a specific strict deadline.

### Core RTOS Concepts

- Multitasking and Concurrency: An ARM Cortex-M4 core can only execute one machine instruction at a time. However, by rapidly swapping out execution threads via an underlying scheduler, it creates an illusion of simultaneous parallel execution.

- The Scheduler: The heart of FreeRTOS. It uses a Preemptive Priority-Based Scheduling Algorithm. The scheduler ensures that the highest-priority task currently in the "Ready" state is always granted CPU execution time. If a higher-priority task transitions to the ready state, the scheduler instantly halts ("preempts") the lower-priority task.

- Tasks: In FreeRTOS, each independent thread of execution is called a Task. Each task is a structured, infinite loop (while(1)) that should never return or exit directly. Every task is given its own isolated stack space in the SRAM memory to hold local variables and CPU register states during context switching.

- The System Tick Timer (SysTick): The heartbeat of the RTOS. It generates a periodic hardware interrupt (typically set to 1 millisecond). On every system tick, the RTOS kernel runs its internal scheduling logic to evaluate task states and switch tasks if required. Crucial STM32 Note: Because FreeRTOS consumes the SysTick timer, the HAL (Hardware Abstraction Layer) base driver must shift its core timebase source to an alternative peripheral timer (e.g., TIM1 or TIM6) to prevent interrupt conflicts.

- CMSIS-RTOS API Layer: ARM provides the Common Microcontroller Software Interface Standard (CMSIS) RTOS wrapper layer. Instead of invoking native FreeRTOS calls directly (like xTaskCreate), we utilize standardized abstraction macros (like osThreadCreate) which render the underlying application code highly portable across varying silicon vendors.

## 4. Architectural Pinout & Clock Configuration

For this experiment, the internal hardware of the microcontroller must be routed safely to handle the RTOS context switching safely:

- Timebase Source Configuration: System Core > SYS > Timebase Source shifted from SysTick to TIM1 (or TIM6) to ensure HAL delays do not block the kernel scheduler.

- Middleware Integration: FREERTOS activated under the CMSIS_V1 interface specification.

- Clock Tree Configuration: System clock set stably to feed the AHB and APB buses through the main Internal/External Phase-Locked Loop (PLL) multiplier block.

## 5. Firmware Implementation Structure

When generating the code from the hardware configuration profile, the system structures the execution blocks inside main.c. Below is the clean execution template demonstrating how tasks are declared, defined, and initiated under the kernel.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

/* Private variables ---------------------------------------------------------*/

// Step 1: Thread Handle definitions for tracking tasks

osThreadId defaultTaskHandle;

osThreadId Task02Handle;

osThreadId Task03Handle;

/* Private function prototypes -----------------------------------------------*/

void StartDefaultTask(void const * argument);

void StartTask02(void const * argument);

void StartTask03(void const * argument);

int main(void)

{

/* MCU Configuration--------------------------------------------------------*/

HAL_Init();         /* Reset of all peripherals, Initializes Flash & Systick. */

SystemClock_Config();/* Configure the system clock */

MX_GPIO_Init();     /* Initialize configured General Purpose I/Os */

/* Create the thread(s) and assign priorities */

/* Definition and Creation of Default Task */

osThreadDef(defaultTask, StartDefaultTask, osPriorityNormal, 0, 128);

defaultTaskHandle = osThreadCreate(osThread(defaultTask), NULL);

/* Definition and Creation of Task 02 (High Priority Execution) */

osThreadDef(Task02, StartTask02, osPriorityAboveNormal, 0, 128);

Task02Handle = osThreadCreate(osThread(Task02), NULL);

/* Definition and Creation of Task 03 (Low Priority Execution) */

osThreadDef(Task03, StartTask03, osPriorityIdle, 0, 128);

Task03Handle = osThreadCreate(osThread(Task03), NULL);

/* Start scheduler - Control is passed completely to the RTOS Kernel */

osKernelStart();

/* We should never reach here as control is now grabbed by the scheduler */

while (1)

{

}

}

/*------------------------- Task Implementations -----------------------------*/

/**

* @brief  Function implementing the defaultTask thread.

* @param  argument: Not used

* @retval None

*/

void StartDefaultTask(void const * argument)

{

for(;;)

{

// Default system housekeeping can go here

osDelay(1000); // Non-blocking RTOS delay puts task to "Blocked" state

}

}

/**

* @brief  Function implementing Task02 (High Priority Task)

* @param  argument: Not used

*/

void StartTask02(void const * argument)

{

for(;;)

{

// Insert execution routine here (e.g., Reading critical sensor values)

osDelay(10); // Relinquishes CPU control to allow other tasks to run

}

}

/**

* @brief  Function implementing Task03 (Low Priority Task)

* @param  argument: Not used

*/

void StartTask03(void const * argument)

{

for(;;)

{

// Insert low-urgency routines here (e.g., System status display logging)

osDelay(500);

}

}

## 6. Procedural Steps for Lab Execution

- Initialize IDE: Open STM32CubeIDE, navigate to File > New > STM32 Project.

- Select Target: Use the Board Selector wizard to pick the NUCLEO-F446RE. Initialize with default peripheral modes.

- Timer Adjustment: Navigate to System Core > SYS and change the Timebase Source parameter to TIM1.

- Enable FreeRTOS Middleware: Go to Middleware > FREERTOS, set the interface version mode to CMSIS_V1.

- Establish Tasks: Under the configuration tab for FreeRTOS, open the Tasks and Queues sub-pane. Click Add to populate Task02 (assign high priority) and Task03 (assign low priority). Keep stack instances at 128 words.

- Code Generation: Save the .ioc configuration canvas to run the Device Configuration Tool and auto-generate the project code scaffolding.

- Refine Logic: Navigate inside src/main.c, track down the task implementation stubs at the footer of the file, and append test indicators or toggle code to track execution.

- Compile & Execute: Build the binary (Ctrl + B), connect the Nucleo hardware using a USB interface cable, and flash the target layout using the debugger module tool.

## 7. Observations & Verification

- Preemptive Interventions: When executing, the kernel tracks task delays accurately. While a high-priority task resides within its osDelay() window, its state transitions to Blocked, immediately granting the underlying low-priority task (Task03) permission to execute.

- Deterministic Timing: Once the time tracking counter expires, the high-priority thread immediately preempts lower tasks, confirming that application urgency levels safely supersede chronological rotation cycles.