Here is the comprehensive GitHub report material template for Experiment 2, structured to follow the same academic standards and format as the previous experiment.

# Experiment 2: Inter-Task Communication using FreeRTOS Queues on STM32F446RE

## 1. Objective

- To understand the concepts of Inter-Task Communication (ITC) and data sharing in a real-time multitasking environment.

- To safely pass data between independent threads without causing race conditions or data corruption.

- To configure, initialize, and implement FreeRTOS Queues on the STM32F446RE Nucleo board using the CMSIS-RTOS V1 API layer.

## 2. Hardware and Software Requirements

- Hardware: * STM32F446RE Nucleo-64 Development Board.

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY) configured to the target baud rate (typically 115200 bps) to observe data transmitted across tasks.

## 3. Theoretical Background

### The Challenge of Data Sharing in RTOS

In a preemptive RTOS, if two tasks attempt to read or modify a shared global variable simultaneously, data corruption can occur due to sudden context switching. To prevent this, an RTOS provides structured synchronization and communication primitives.

### FreeRTOS Queues

A Queue is the primary mechanism for sending data between tasks. It acts as a thread-safe FIFO (First-In, First-Out) buffer managed entirely by the RTOS kernel.

### Key Characteristics of Queues:

- Thread Safety: Queues automatically handle mutual exclusion. The kernel ensures that only one task can access the queue data at any given moment.

- Blocking on Queue Access: * Queue Read (Receive): If a task tries to read from an empty queue, it can enter the Blocked state, relinquishing CPU execution time until data arrives or a timeout expires.

  - Queue Write (Send): If a task tries to write to a full queue, it can block until space becomes available.

- Pass-by-Value vs. Pass-by-Reference: FreeRTOS queues copy data by value into the queue buffer. This means if you pass a local integer variable, its actual content is copied, preventing memory access issues if the sending task changes the original variable later.

## 4. Architectural Configuration in STM32CubeMX

To configure a project for Queue communication:

- Timebase Source: Ensure System Core > SYS > Timebase Source remains set to an alternative timer (e.g., TIM1) so that HAL delays don't conflict with FreeRTOS.

- Middleware Integration: Under FREERTOS, navigate to the Tasks and Queues tab.

- Queue Creation: Under the "Queues" table, select Add.

  - Set the Queue Name (e.g., myQueue01Handle).

  - Define the Queue Size (e.g., 16 elements).

  - Select the Item Size (e.g., uint32_t or a custom data structure).

## 5. Firmware Implementation Structure

The following code demonstrates a classic Producer-Consumer design pattern. ProducerTask creates a data payload and pushes it into the queue, while ConsumerTask blocks until data arrives, extracts it, and routes it to the UART interface.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Private variables ---------------------------------------------------------*/

// Handles for our communicating tasks

osThreadId ProducerTaskHandle;

osThreadId ConsumerTaskHandle;

// Handle for the Inter-Task Communication Queue

osMessageQId myQueue01Handle;

/* UART handle for serial monitoring output */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartProducerTask(void const * argument);

void StartConsumerTask(void const * argument);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

/* --- Create the Queue --- */

// Defines a queue capable of holding 16 elements, each of size uint32_t

osMessageQDef(myQueue01, 16, uint32_t);

myQueue01Handle = osMessageCreate(osMessageQ(myQueue01), NULL);

/* --- Create the Threads --- */

/* Definition and Creation of the Producer Task */

osThreadDef(ProducerTask, StartProducerTask, osPriorityNormal, 0, 128);

ProducerTaskHandle = osThreadCreate(osThread(ProducerTask), NULL);

/* Definition and Creation of the Consumer Task */

osThreadDef(ConsumerTask, StartConsumerTask, osPriorityNormal, 0, 128);

ConsumerTaskHandle = osThreadCreate(osThread(ConsumerTask), NULL);

/* Start the RTOS Scheduler */

osKernelStart();

while (1)

{

// The kernel controls execution; code execution should never loop here.

}

}

/*------------------------- Task Implementations -----------------------------*/

/**

* @brief  Producer Task: Generates data increments and posts them to the queue.

*/

void StartProducerTask(void const * argument)

{

uint32_t producer_count = 0;

for(;;)

{

producer_count++;

// Send data to the queue. Timeout is set to 0 (non-blocking write)

osStatus status = osMessagePut(myQueue01Handle, producer_count, 0);

if(status == osOK)

{

// Data successfully posted to the queue buffer

}

// Generate data every 500 milliseconds

osDelay(500);

}

}

/**

* @brief  Consumer Task: Blocks until data is available in the queue, then logs it.

*/

void StartConsumerTask(void const * argument)

{

osEvent event;

char uart_buf[50];

for(;;)

{

// Block indefinitely (osWaitForever) until an item arrives in the queue

event = osMessageGet(myQueue01Handle, osWaitForever);

if(event.status == osEventMessage)

{

// Extract the sent value from the event container

uint32_t received_val = event.value.v;

// Print the synchronized data out via UART

int len = snprintf(uart_buf, sizeof(uart_buf), "Queue Received Data: %lu\r\n", received_val);

HAL_UART_Transmit(&huart2, (uint8_t*)uart_buf, len, HAL_MAX_DELAY);

}

}

}

## 6. Procedural Steps for Lab Execution

- Open Existing Workspace: Launch STM32CubeIDE and open your baseline configuration project.

- Configure Middleware Device Engine: Double-click the project .ioc file. Navigate to Middleware > FREERTOS.

- Build the Queue Infrastructure: Open the Tasks and Queues tab. Scroll down to the Queues section, click Add, allocate the configuration attributes (myQueue01, Allocation size: 16, Data Type: uint32_t), and click OK.

- Define Operating Threads: Ensure you have two tasks declared: ProducerTask and ConsumerTask, keeping their operational priority balanced.

- Generate Code Layout: Press Ctrl + S to save and auto-generate the updated code block architecture.

- Integrate Application Code: Open main.c. Add the logic inside StartProducerTask to push increments via osMessagePut(), and update StartConsumerTask to handle the data payloads using osMessageGet().

- Compile and Run: Build your binaries, flash the code onto the board, and launch your serial monitor terminal to trace the console results.

## 7. Observations & Verification

- Data Flow Logging: Upon resetting the hardware, the serial terminal output displays sequential increments cleanly at exactly 500ms gaps, verifying that data successfully transitions across isolated execution loops.

- Deterministic Synchronization: Because the Consumer task is set to osWaitForever, it rests in a highly efficient Blocked state, consuming zero CPU cycles while waiting. The exact millisecond the Producer puts a value into the queue, the kernel immediately transitions the Consumer to the Running state to read the message.