Here is the comprehensive GitHub report material template for Experiment 3, focusing on task synchronization using Mutexes and Semaphores, structured to match the academic formatting of the previous experiments.

# Experiment 3: Task Synchronization using Mutexes and Semaphores on STM32F446RE

## 1. Objective

- To understand the concepts of resource contention, race conditions, and critical sections in a preemptive multitasking environment.

- To learn how to safely synchronize tasks and protect shared hardware or memory resources using Mutexes.

- To implement signaling mechanisms between multiple execution threads using Binary/Counting Semaphores on the STM32F446RE Nucleo board using the CMSIS-RTOS V1 API layer.

## 2. Hardware and Software Requirements

- Hardware: * STM32F446RE Nucleo-64 Development Board.

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY) to observe task execution timelines and serial outputs.

## 3. Theoretical Background

### The Shared Resource Challenge

When multiple tasks running at different priority levels attempt to access the exact same peripheral or memory space (such as a shared SPI bus, an LCD display, or a global variable), it creates a Critical Section. If a low-priority task is halfway through writing data to a peripheral and gets preempted by a high-priority task attempting to write to that same peripheral, the data corrupts. This scenario is called a Race Condition.

### Mutex (Mutual Exclusion)

A Mutex is a specialized binary token used strictly for Resource Protection. It acts like a lock:

- A task must "Take" (lock) the Mutex before accessing the shared resource.

- If another task tries to take the Mutex while it is locked, that task is placed into a Blocked state by the scheduler.

- Once the owning task completes its critical section, it must "Give" (unlock) the Mutex back, allowing waiting tasks to access the resource.

#### Priority Inversion and Priority Inheritance

A critical problem with standard synchronization is Priority Inversion: a high-priority task is blocked waiting for a resource held by a low-priority task, while a medium-priority task (which doesn't even use the resource) keeps preempting the low-priority task, indefinitely delaying the high-priority task. FreeRTOS Mutexes implement Priority Inheritance to solve this: the low-priority task holding the Mutex temporarily inherits the high priority of the waiting task, allowing it to finish its work quickly and release the lock.

### Semaphores

While a Mutex implies ownership (the task that takes it must give it back), a Semaphore is used for Task Signaling (one task signals another task to run).

- Binary Semaphore: Has a maximum count of 1. It acts like a single flag. Task A can wait for a signal (osSemaphoreWait), and an external Interrupt Service Routine (ISR) or Task B can drop the flag (osSemaphoreRelease) to wake it up.

- Counting Semaphore: Has a maximum count greater than 1. It is used to manage a pool of available resources or keep track of counting events.

## 4. Architectural Configuration in STM32CubeMX

To configure synchronization primitives under FreeRTOS:

- Timebase Source: Ensure System Core > SYS > Timebase Source remains assigned to TIM1 or TIM6.

- Middleware Integration: Under FREERTOS, navigate to the configuration layout tabs.

- Mutex Configuration: Open the Mutexes tab, click Add, and give your mutex an identifier (e.g., myMutex01).

- Semaphore Configuration: Alternatively, open the Semaphores tab to define Binary or Counting tokens depending on your design logic.

## 5. Firmware Implementation Structure

The following architectural template implements a Mutex lock protecting the Shared UART2 hardware peripheral. Two tasks with conflicting timing schedules attempt to write to the console concurrently, but their outputs remain clean and uncorrupted due to the synchronization lock.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Private variables ---------------------------------------------------------*/

// Task IDs

osThreadId HighPriorityTaskHandle;

osThreadId LowPriorityTaskHandle;

// Mutex ID for managing resource ownership

osMutexId myMutex01Handle;

/* UART handle for console print streaming */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartHighPriorityTask(void const * argument);

void StartLowPriorityTask(void const * argument);

void SafePrint(const char* message);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

/* --- Create the Mutex Object --- */

osMutexDef(myMutex01);

myMutex01Handle = osMutexCreate(osMutex(myMutex01));

/* --- Create Operating Threads --- */

/* High Priority Task */

osThreadDef(HighPriorityTask, StartHighPriorityTask, osPriorityAboveNormal, 0, 128);

HighPriorityTaskHandle = osThreadCreate(osThread(HighPriorityTask), NULL);

/* Low Priority Task */

osThreadDef(LowPriorityTask, StartLowPriorityTask, osPriorityBelowNormal, 0, 128);

LowPriorityTaskHandle = osThreadCreate(osThread(LowPriorityTask), NULL);

/* Start RTOS Scheduler */

osKernelStart();

while (1)

{

// Kernel controlled loop

}

}

/*------------------------- Thread Implementations ---------------------------*/

/**

* @brief  Thread safe wrapper function to protect the UART hardware resource.

*/

void SafePrint(const char* message)

{

// Block indefinitely until the Mutex token is free to take

if (osMutexWait(myMutex01Handle, osWaitForever) == osOK)

{

// CRITICAL SECTION START: Exclusive ownership of huart2

int len = 0;

while(message[len] != '\0') len++;

HAL_UART_Transmit(&huart2, (uint8_t*)message, len, HAL_MAX_DELAY);

// CRITICAL SECTION END

// Release the resource token back to the kernel pool

osMutexRelease(myMutex01Handle);

}

}

/**

* @brief  High Priority Task: Runs frequently and requests resource access.

*/

void StartHighPriorityTask(void const * argument)

{

for(;;)

{

SafePrint("CRITICAL MESSAGE: High Priority Task working...\r\n");

osDelay(100); // Sleep for 100ms

}

}

/**

* @brief  Low Priority Task: Runs less frequently but transmits dense data blocks.

*/

void StartLowPriorityTask(void const * argument)

{

for(;;)

{

SafePrint("System status report: Low Priority processing complete data block.\r\n");

osDelay(300); // Sleep for 300ms

}

}

## 6. Procedural Steps for Lab Execution

- Open CubeMX File: Launch your workspace project architecture mapping workspace within STM32CubeIDE.

- Enable Synchronization Primitive: Double-click the project .ioc configuration tree. Head to Middleware > FREERTOS.

- Initialize Mutex Memory: Under the Mutexes tab, click Add, naming the new instance handle myMutex01.

- Task Definitions: Ensure two tasks are defined with distinct, staggered priority assignments (e.g., osPriorityAboveNormal vs osPriorityBelowNormal).

- Code Re-Generation: Save your configurations (Ctrl + S) to automatically insert the structural RTOS initializations into the code body.

- Implement Code Blocks: Embed the wrapper resource protection logic (osMutexWait and osMutexRelease) inside the printing routing profiles within main.c.

- Compile, Flash, and Trace: Build the binaries, execute them on the Nucleo processor, and capture the serialization data streams cleanly.

## 7. Observations & Verification

- Data Stream Integrity: If the Mutex protection calls are stripped out, the serial console outputs interweave, creating garbled text strings during periods when the high-priority task preempts the low-priority task mid-transmission.

- Deterministic Mutual Exclusion: When the Mutex is integrated, the text lines print sequentially with flawless formatting. If the low-priority task holds the lock, the high-priority task is cleanly forced into a Blocked state by the scheduler until the low-priority task completely relinquishes ownership via the osMutexRelease() call.