Here is the comprehensive GitHub report material template for Experiment 10, focusing on Low Power Optimization and the Tickless Idle Mode, structured to match the academic formatting of the previous experiments.

# Experiment 10: Low Power Optimization and Tickless Idle Mode in FreeRTOS on STM32F446RE

## 1. Objective

- To understand the impact of an active RTOS kernel scheduler on the power consumption profile of an embedded microcontroller.

- To analyze the mechanics of the FreeRTOS Tickless Idle Mode and how it interfaces with the low-power architectures of the ARM Cortex-M4 core.

- To implement, configure, and evaluate low-power entry and exit behaviors on the STM32F446RE Nucleo board using the CMSIS-RTOS V1 API layer.

## 2. Hardware and Software Requirements

- Hardware:

  - STM32F446RE Nucleo-64 Development Board.

  - Digital Multimeter / Current Ammeter (optional, for measuring physical power reduction).

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY).

## 3. Theoretical Background

### The Power Dilemma in Traditional RTOS

In a standard FreeRTOS application, the periodic System Tick Timer (SysTick) generates a hardware interrupt exactly every 1 millisecond to increment the tick count and evaluate task states.

Even if all application tasks are completely blocked (waiting for a future delay or event), the microcontroller is continuously forced to wake up 1,000 times per second just to service the SysTick interrupt. This continuous cycle of waking up and running the scheduler keeps the CPU core active, preventing the microcontroller from entering deep, prolonged sleep modes, which quickly drains batteries in remote or portable applications.

### FreeRTOS Tickless Idle Mode

To solve this issue, FreeRTOS includes a feature called Tickless Idle Mode (configUSE_TICKLESS_IDLE).

When the scheduler determines that all application tasks are blocked and the next scheduled task will not wake up for several milliseconds, it shuts down the standard SysTick interrupt. Instead, it calculates the exact duration the system can safely sleep and configures a low-power hardware timer to wake the CPU up right before that next task deadline.

### ARM Cortex-M4 Sleep Architecture & STM32 Power Modes

During the tickless idle window, the kernel executes the WFI (Wait For Interrupt) assembly command, transitioning the ARM core into one of the STM32's native low-power states:

- Sleep Mode: The CPU clock is stopped, but all peripherals and internal clocks keep running. Waking up is almost instant.

- Stop Mode: All high-speed clocks (HSI, HSE, PLL) are disabled. The CPU and most peripherals lose their clocks, but RAM contents are preserved. Waking up takes a few microseconds while the PLL stabilizes.

- Standby Mode: The deepest sleep mode. The entire internal core voltage domain is switched off. All RAM contents are lost, and waking up causes a full system reset.

For standard Tickless Idle, Sleep Mode or a shallow Stop Mode is typically preferred to ensure that the kernel can wake up quickly without missing task execution deadlines.

## 4. Architectural Configuration in STM32CubeMX

To configure your project workspace for low-power operation:

- Timebase Source Alteration: Ensure System Core > SYS > Timebase Source is set to an alternate peripheral timer (e.g., TIM1 or TIM6) so that the basic HAL tick layer does not keep waking up the MCU independently of the FreeRTOS core.

- Middleware Integration: Under FREERTOS, navigate to the Config parameters tab.

  - Locate configUSE_TICKLESS_IDLE and change it to Enabled.

- Pin Configuration: For optimal power savings in production, any unused GPIO pins should be configured as Analog Input to stop internal digital input buffers from consuming current due to floating voltage levels.

## 5. Firmware Implementation Structure

The following structural template demonstrates how to implement low-power hook configurations. It creates a task that sleeps for long durations, allowing the system to drop into Tickless Idle, and defines the pre/post-sleep processor configurations.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Private variables ---------------------------------------------------------*/

// Thread Handle for the extended periodic low power sleeper task

osThreadId PeriodicSleeperTaskHandle;

/* UART handle for power log notifications */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartPeriodicSleeperTask(void const * argument);

// Custom overrides for Low Power entry and exit hooks called by the kernel

void PreSleepProcessing(uint32_t *ulExpectedIdleTime);

void PostSleepProcessing(uint32_t *ulExpectedIdleTime);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

char msg_init[] = "--- Low Power Optimization & Tickless Idle Active ---\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_init, sizeof(msg_init)-1, 100);

/* --- Create the Extended Sleep Task --- */

osThreadDef(PeriodicSleeper, StartPeriodicSleeperTask, osPriorityNormal, 0, 128);

PeriodicSleeperTaskHandle = osThreadCreate(osThread(PeriodicSleeper), NULL);

/* Start RTOS Scheduler */

osKernelStart();

while (1)

{

// Kernel controlled loop

}

}

/*------------------------- Low Power Hook Implementations --------------------*/

/**

* @brief  Pre-Sleep Processing Callback. Called by FreeRTOS right before

* the MCU stops its clocks and enters sleep mode.

*/

void PreSleepProcessing(uint32_t *ulExpectedIdleTime)

{

/* OPTIONAL: Turn off non-essential peripheral clocks, status LEDs,

or place external transceivers into low-power mode here. */

// Example: Turn off the onboard Green LED to verify sleep physically

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_RESET);

}

/**

* @brief  Post-Sleep Processing Callback. Called by FreeRTOS the instant

* the MCU wakes back up from its low-power state.

*/

void PostSleepProcessing(uint32_t *ulExpectedIdleTime)

{

/* Re-enable any peripheral clocks or pin states disabled in PreSleep.

If waking up from Stop mode, re-configure system clocks/PLL here. */

// Example: Turn back on the onboard Green LED

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_SET);

}

/*------------------------- Task Implementations -----------------------------*/

/**

* @brief  Periodic Sleeper Task: Performs brief work and then requests

* a long sleep window, triggering the Tickless Idle state.

*/

void StartPeriodicSleeperTask(void const * argument)

{

uint32_t loop_counter = 0;

char msg_buf[60];

for(;;)

{

loop_counter++;

// Perform quick activity (e.g., log progress)

int len = snprintf(msg_buf, sizeof(msg_buf), "[ACTIVE] Working on loop execution: %lu\r\n", loop_counter);

HAL_UART_Transmit(&huart2, (uint8_t*)msg_buf, len, 100);

// Toggle LED to show active state before dropping to sleep

HAL_GPIO_WritePin(GPIOA, GPIO_PIN_5, GPIO_PIN_SET);

osDelay(100); // Wait 100ms in an active operational cycle

/* CRITICAL STEP: Enter a long blocking delay (e.g., 5 seconds).

Because no other tasks are ready, FreeRTOS will automatically see this

and shift the MCU core into Tickless Idle Mode. */

int len_sleep = snprintf(msg_buf, sizeof(msg_buf), "Entering Deep Tickless Sleep for 5000ms...\r\n\r\n");

HAL_UART_Transmit(&huart2, (uint8_t*)len_sleep, len_sleep, 100);

osDelay(5000);

}

}

## 6. Procedural Steps for Lab Execution

- Initialize Base Workspace: Open your ongoing project environment configurations in STM32CubeIDE.

- Enable Tickless Idle Mode: Open the project .ioc configuration view. Navigate to Middleware > FREERTOS > Config parameters. Locate the variable entry for configUSE_TICKLESS_IDLE and switch it to Enabled (Built-in functionality).

- Register Threads: Create a target testing thread (PeriodicSleeper) inside your Tasks settings panel.

- Generate Code Scaffolding: Save the configuration specifications (Ctrl + S) to update the project initialization code and pull in the low-power hooks.

- Program the Low Power Routines: Open main.c. Implement your application task with a long osDelay(5000) call block. Fill in the PreSleepProcessing and PostSleepProcessing code structures to manage peripherals (like turning off LEDs or UART channels) during power transitions.

- Compile and Execute: Build the binary definitions, upload the firmware to your target board, and launch your serial interface module.

- Verify Power Drops (Hardware Check): If a multimeter is available, break the Nucleo board's IDD jumper link and place an ammeter across the pins to measure the drop in current consumption during the 5-second sleep window.

## 7. Observations & Verification

- Functional Log Tracing: The serial logging stream cleanly outputs its [ACTIVE] state message, prints its intention to sleep, and then completely pauses transmissions for exactly 5000 ms before waking up to repeat the loop.

- Physical Indicator Syncing: The onboard Green LED stays lit for just 100 ms during active processing, then immediately turns off for 5 seconds when the system enters the PreSleepProcessing callback loop. This confirms that the RTOS is successfully bypassing standard 1 ms SysTick wakeups and keeping the MCU core asleep during long idle windows.

- Efficiency Analysis: In a normal configuration, the MCU continuously draws full operational current (typically ~20–40 mA) to process the 1 ms system tick. Enabling Tickless Idle allows current consumption to drop significantly during sleep windows, proving that this mode is an essential technique for designing efficient, battery-powered real-time embedded systems.