Here is the comprehensive GitHub report material template for Experiment 8, focusing on Task Priorities, Preemption, and Starvation, structured to match the academic formatting of the previous experiments.

# Experiment 8: Task Priorities, Preemption, and Starvation in FreeRTOS on STM32F446RE

## 1. Objective

- To investigate the behavior of the FreeRTOS Preemptive Priority-Based Scheduler.

- To analyze how tasks with different priorities interact, preempt each other, and share CPU execution time.

- To simulate, observe, and mitigate the classic real-time computing issue of Task Starvation.

## 2. Hardware and Software Requirements

- Hardware:

  - STM32F446RE Nucleo-64 Development Board.

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY).

## 3. Theoretical Background

### Preemptive Priority-Based Scheduling

FreeRTOS relies primarily on a preemptive scheduler. Each task is assigned a specific priority level upon creation.

- Preemption: If a task with a priority higher than the currently running task transitions from the Blocked or Suspended state into the Ready state, the scheduler instantly halts the lower-priority task and context-switches to the higher-priority task.

- Time-Slicing (Round-Robin): If multiple tasks share the exact same priority level, the scheduler alternates execution between them on every millisecond system tick interrupt, ensuring they share CPU time equally.

### The Phenomenon of Task Starvation

In a purely preemptive scheduling model, a low-priority task can only run if no higher-priority tasks are ready.

If a high-priority task contains an unblocked, infinite computational loop—or if it executes and clears its delay so fast that it never gives up control—lower-priority tasks will never receive CPU time. This condition is known as Starvation. The lower-priority tasks are effectively frozen, starving for processing cycles.

### Mitigation Strategies

To prevent starvation in a multi-tasking design, developers must implement structural safeguards:

- Enforce Yielding via Blocking Delays: High-priority tasks must periodically place themselves into a Blocked state using non-blocking delays (osDelay()) or wait conditions on synchronization flags (queues, semaphores), opening up execution windows for lower priority tiers.

- Manual Yielding (osThreadYield): A task can explicitly signal the scheduler to look for other tasks of equal priority. However, this does not allow lower-priority tasks to break in.

- Dynamic Priority Adjustment: Temporarily boosting the starved task's priority level at runtime to pull it out of a lock context.

## 4. Architectural Configuration in STM32CubeMX

To configure a project to safely test priority behaviors:

- Timebase Source: Ensure System Core > SYS > Timebase Source is set to an alternate timer (e.g., TIM1) so that underlying HAL delays do not disrupt scheduler preemption testing.

- Middleware Integration: Under FREERTOS, navigate to the Tasks and Queues tab.

- Establish Imbalanced Threads:

  - Define HighPriorityTask with an elevated priority level (e.g., osPriorityAboveNormal).

  - Define LowPriorityTask with a diminished priority level (e.g., osPriorityBelowNormal).

## 5. Firmware Implementation Structure

The following code pattern demonstrates an intentional starvation scenario and its mitigation. The HighPriorityTask runs an intensive calculation loop. If the STARVATION_TEST macro is enabled, it completely starves the LowPriorityTask. If it is disabled, the high-priority task safely yields, allowing both tasks to execute.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Configuration Macro -------------------------------------------------------*/

// Set to 1 to induce Starvation. Set to 0 to enable Mitigation (osDelay).

#define ENFORCE_STARVATION_MODE   1

/* Private variables ---------------------------------------------------------*/

// Task Handles

osThreadId HighPriorityTaskHandle;

osThreadId LowPriorityTaskHandle;

/* UART handle for execution tracing */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartHighPriorityTask(void const * argument);

void StartLowPriorityTask(void const * argument);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

char msg_start[] = "--- Scheduling & Priority Experiment Starting ---\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_start, sizeof(msg_start)-1, 100);

/* --- Create the Low Priority Task --- */

osThreadDef(LowTask, StartLowPriorityTask, osPriorityBelowNormal, 0, 128);

LowPriorityTaskHandle = osThreadCreate(osThread(LowTask), NULL);

/* --- Create the High Priority Task --- */

osThreadDef(HighTask, StartHighPriorityTask, osPriorityAboveNormal, 0, 128);

HighPriorityTaskHandle = osThreadCreate(osThread(HighTask), NULL);

/* Start RTOS Scheduler */

osKernelStart();

while (1)

{

// Kernel controlled loop

}

}

/*------------------------- Task Implementations -----------------------------*/

/**

* @brief  High Priority Task: Demonstrates CPU consumption and scheduler control.

*/

void StartHighPriorityTask(void const * argument)

{

char msg_high[] = "[HIGH PRIORITY] Task is executing...\r\n";

for(;;)

{

HAL_UART_Transmit(&huart2, (uint8_t*)msg_high, sizeof(msg_high)-1, 100);

#if ENFORCE_STARVATION_MODE

// STARVATION MODE: Heavy, unblocked computational loop.

// This keeps the task constantly in the "Running" or "Ready" state.

for(volatile uint32_t i = 0; i < 600000; i++);

// Because there is no blocking delay here, the low-priority task starves.

#else

// MITIGATION MODE: Introducing a blocking RTOS delay.

// This forces the high-priority task into the "Blocked" state, freeing the CPU.

osDelay(500);

#endif

}

}

/**

* @brief  Low Priority Task: Dependent on higher priority tasks yielding control.

*/

void StartLowPriorityTask(void const * argument)

{

char msg_low[] = "--> [LOW PRIORITY] Task safely received CPU cycles!\r\n";

for(;;)

{

// Print notification when CPU access is granted

HAL_UART_Transmit(&huart2, (uint8_t*)msg_low, sizeof(msg_low)-1, 100);

// Small delay to prevent terminal output flooding

osDelay(100);

}

}

## 6. Procedural Steps for Lab Execution

- Open Existing Project Base: Launch your ongoing FreeRTOS configuration workspace inside STM32CubeIDE.

- Configure Multi-priority Threads: Double-click the project .ioc layout tree. Under Middleware > FREERTOS, access the Tasks and Queues layout panel.

- Set Asymmetric Priorities: Add HighTask set to osPriorityAboveNormal and LowTask assigned to osPriorityBelowNormal.

- Generate Code Layout: Save the configuration changes (Ctrl + S) to update the project initialization variables.

- Implement Starvation Scenario: In main.c, set the ENFORCE_STARVATION_MODE macro configuration flag to 1. Complete the code loop blocks for both functions.

- Compile and Run (Test 1): Build and flash the project onto your board. Open the serial communication terminal and observe the output behavior.

- Apply Mitigation (Test 2): Modify the ENFORCE_STARVATION_MODE macro switch to 0. Recompile the firmware code, reload it onto the target microcontroller, and evaluate the changes in the serial log stream.

## 7. Observations & Verification

### Case 1: Starvation Mode enabled (ENFORCE_STARVATION_MODE 1)

When the system boots up, the serial terminal displays a continuous, repeating stream of:

Plaintext

[HIGH PRIORITY] Task is executing...

[HIGH PRIORITY] Task is executing...

[HIGH PRIORITY] Task is executing...

- Observation: The Low Priority task trace message never prints out. Because the high-priority thread never transitions into a Blocked state, the scheduler never checks the lower priority ready queue, causing complete Task Starvation.

### Case 2: Mitigation Mode enabled (ENFORCE_STARVATION_MODE 0)

Once the macro is switched to 0 and an osDelay() is introduced, the terminal output shifts to a balanced, interleaved pattern:

Plaintext

[HIGH PRIORITY] Task is executing...

--> [LOW PRIORITY] Task safely received CPU cycles!

--> [LOW PRIORITY] Task safely received CPU cycles!

[HIGH PRIORITY] Task is executing...

- Observation: The moment the high-priority task hits osDelay(500), it willingly yields control and enters the Blocked state. The scheduler instantly drops down to run the Low Priority task. This confirms that proper timing design and structured delays are essential for predictable, cooperative multitasking in real-time embedded systems.