Here is the comprehensive GitHub report material template for Experiment 5, focusing on Interrupt Management and Deferred Interrupt Processing (Bottom-Half Processing), structured to match the academic formatting of the previous experiments.

# Experiment 5: Interrupt Management and Deferred Interrupt Processing on STM32F446RE

## 1. Objective

- To understand the interaction between hardware interrupts (ISRs) and a Real-Time Operating System kernel.

- To learn the limitations of executing long-running code inside an ISR and master the concept of Deferred Interrupt Processing.

- To implement thread-safe signaling from a hardware ISR to an RTOS task using FromISR API variants via the CMSIS-RTOS V1 layer on the STM32F446RE Nucleo board.

## 2. Hardware and Software Requirements

- Hardware:

  - STM32F446RE Nucleo-64 Development Board.

  - Onboard User Push Button (Blue button, mapped to GPIO Pin PC13).

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY).

## 3. Theoretical Background

### The ISR Limitation in RTOS

Hardware interrupts (ISRs) on the ARM Cortex-M4 architecture are prioritatically higher than any software task managed by the FreeRTOS scheduler. If an ISR takes too long to execute (e.g., waiting for a slow sensor read or handling a dense print routine), it halts the RTOS scheduler ticker, locks up lower-priority tasks, and compromises system determinism.

Rule of Thumb: Keep ISRs as short and fast as possible. Never use blocking delays (HAL_Delay or osDelay) or intensive operations inside an interrupt handler.

### Deferred Interrupt Processing (Top/Bottom Half)

To keep ISR execution times minimal, systems use a pattern called Deferred Interrupt Processing:

- Top Half (The ISR): The physical hardware interrupt triggers. It does the absolute minimum work required (e.g., clears the hardware interrupt pending flag, captures a rapid register value) and immediately signals an RTOS task.

- Bottom Half (The Deferred Task): A standard RTOS task blocks, waiting for that specific signal. The moment the signal arrives from the ISR, the task wakes up and completes the heavy processing (e.g., parsing data packets or updating a display) at its own software priority level.

### The "FromISR" Rule

You cannot call standard FreeRTOS API functions (like osSemaphoreRelease) inside a hardware interrupt. If an interrupt triggers a context switch while the kernel is altering its internal structures, the OS will crash. FreeRTOS provides specialized, deterministic FromISR variants (e.g., xSemaphoreGiveFromISR) designed specifically to be safe within execution states where the scheduler is temporarily suspended.

## 4. Architectural Configuration in STM32CubeMX

To bridge physical pin interrupts to the RTOS layer:

- Pin Assignment: Configure GPIO Pin PC13 (connected to the Blue User Button) as GPIO_EXTI13 (External Interrupt Mode with Rising/Falling edge detection).

- NVIC Configuration: Navigate to System Core > NVIC. Check the box to enable the EXTI line[15:10] interrupts.

- Priority Sub-grouping: Ensure that the preemption priority of the EXTI interrupt is logically managed. In FreeRTOS, any interrupt using RTOS API calls must have its numerical priority value equal to or greater than the configMAX_SYSCALL_INTERRUPT_PRIORITY setting (lower physical hardware priority) to prevent kernel corruption.

- Middleware Integration: Under FREERTOS, define a binary semaphore or a tracking task that will capture the upcoming hardware signal event.

## 5. Firmware Implementation Structure

The following code pattern demonstrates safe deferred processing. Pressing the blue user button triggers a hardware EXTI interrupt, which instantly unblocks a processing handler task via an RTOS Semaphore signal.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Private variables ---------------------------------------------------------*/

// Task Handle for the Deferred Processing Task

osThreadId DeferredTaskHandle;

// Binary Semaphore Handle for ISR-to-Task Signaling

osSemaphoreId bSemaphoreISRHandle;

/* UART handle for tracking logs */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartDeferredTask(void const * argument);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init(); // This initializes PC13 as EXTI line input

MX_USART2_UART_Init();

/* --- Create the Binary Semaphore --- */

osSemaphoreDef(bSemaphoreISR);

bSemaphoreISRHandle = osSemaphoreCreate(osSemaphore(bSemaphoreISR), 1);

// Ensure the semaphore starts in an empty state (0 tokens available)

osSemaphoreWait(bSemaphoreISRHandle, 0);

/* --- Create the Deferred Task --- */

// Give it a high priority so it executes immediately after the ISR completes

osThreadDef(DeferredTask, StartDeferredTask, osPriorityHigh, 0, 128);

DeferredTaskHandle = osThreadCreate(osThread(DeferredTask), NULL);

/* Start RTOS Scheduler */

osKernelStart();

while (1)

{

// Kernel controlled loop

}

}

/*------------------------- Interrupt & Task Implementations -----------------*/

/**

* @brief  EXTI Line Detection Callback (The Top Half).

* Triggered whenever GPIO Pin PC13 experiences a falling edge.

*/

void HAL_GPIO_EXTI_Callback(uint16_t GPIO_Pin)

{

if(GPIO_Pin == GPIO_PIN_13)

{

// High Priority Task Woken flag used by FreeRTOS to evaluate instant context switches

BaseType_t xHigherPriorityTaskWoken = pdFALSE;

/* Unblock the deferred processing task using the FromISR API wrapper.

CMSIS-RTOS V1 translates native FreeRTOS calls behind the scenes. */

xSemaphoreGiveFromISR(bSemaphoreISRHandle, &xHigherPriorityTaskWoken);

/* If the deferred task has a higher priority than the task that was

running before the interrupt, force a context switch right out of the ISR. */

portYIELD_FROM_ISR(xHigherPriorityTaskWoken);

}

}

/**

* @brief  Deferred Processing Task (The Bottom Half).

* Blocks efficiently until signaled by the hardware ISR callback.

*/

void StartDeferredTask(void const * argument)

{

char msg_wait[] = "Deferred Task: Sleeping, waiting for hardware EXTI trigger...\r\n";

char msg_exec[] = "[ISR EVENT DEFERRED] Button Press detected! Executing heavy computations...\r\n";

for(;;)

{

HAL_UART_Transmit(&huart2, (uint8_t*)msg_wait, sizeof(msg_wait)-1, 100);

// Block indefinitely until the hardware interrupt releases a semaphore token

if (osSemaphoreWait(bSemaphoreISRHandle, osWaitForever) == osOK)

{

// Execute the "heavy lifting" code here at software level instead of inside the ISR

HAL_UART_Transmit(&huart2, (uint8_t*)msg_exec, sizeof(msg_exec)-1, 100);

// Simulate heavy data computation or debouncing wait

osDelay(250);

}

}

}

## 6. Procedural Steps for Lab Execution

- Configure Hardware Pin Vectors: Open your project .ioc matrix file. Locate pin PC13, click it, and reassign it to GPIO_EXTI13.

- Turn on the Global NVIC Vector: Under System Core > NVIC, scroll down to track the EXTI line interrupt. Tick the enabled box checkbox.

- Set Up the Signaling Semaphore: Move to the FreeRTOS Middleware settings tab. In the Binary Semaphores section, create a handle called bSemaphoreISR.

- Task Assignment: Add DeferredTask under the Tasks panel, and ensure its priority level is high (osPriorityHigh).

- Generate Project Framework: Save your work (Ctrl + S) to run the automated compilation configuration engines.

- Integrate Application Routine: Open main.c. Scroll down to the bottom of the file or user-code sections and manually implement the overridden HAL_GPIO_EXTI_Callback() template. Fill in the task function logic inside StartDeferredTask.

- Compile, Flash, and Debug: Build the project code configuration binaries, upload them to your hardware target, and inspect the event behavior using your UART terminal.

## 7. Observations & Verification

- Immediate System Responsiveness: When the user button is pressed, the log statement inside StartDeferredTask prints immediately, demonstrating near-zero latency between physical contact and software awareness.

- Deterministic Stability: Because the code uses portYIELD_FROM_ISR(), the system doesn't wait around for the next periodic millisecond system tick to process the event. Instead, the scheduler executes the deferred task the exact microsecond the hardware interrupt leaves the execution context, maintaining an optimal balance of fast interrupt speeds and safe background processing.