Here is the comprehensive GitHub report material template for Experiment 6, focusing on Memory Management and Stack Overflow Detection, structured to match the academic formatting of the previous experiments.

# Experiment 6: Memory Management and Stack Overflow Detection in FreeRTOS on STM32F446RE

## 1. Objective

- To understand how an RTOS manages Dynamic Memory Allocation (Heap) for tasks, queues, and synchronization primitives.

- To analyze the differences between FreeRTOS Heap Allocation schemes (heap_1 to heap_5).

- To implement and test Stack Overflow Detection hooks on the STM32F446RE Nucleo board to safeguard against memory corruption.

## 2. Hardware and Software Requirements

- Hardware:

  - STM32F446RE Nucleo-64 Development Board.

  - Mini-B USB Cable.

- Software:

  - STM32CubeIDE.

  - Serial Monitor Client (e.g., TeraTerm or PuTTY) to catch panic/debugging logs.

## 3. Theoretical Background

### RTOS Memory Architecture: Stack vs. Heap

In an RTOS environment, memory is divided into two vital allocations:

- The Heap: A large pool of memory allocated globally. When you create a task, a queue, or a semaphore dynamically, the FreeRTOS kernel allocates the required control structures directly out of this heap space.

- The Task Stack: Every individual task has its own isolated stack space allocated from the heap during creation. This stack holds the task's local variables, function call parameters, and a snapshot of the CPU registers during a context switch.

### FreeRTOS Heap Management Schemes

FreeRTOS provides five distinct memory allocation implementations (heap_1.c to heap_5.c). STM32CubeMX defaults to heap_4.c for modern microcontrollers.

- heap_1: The simplest scheme. It allows memory allocation but never allows memory deletion. It is entirely deterministic and safe from memory fragmentation.

- heap_2: Allows allocation and deletion, but does not combine adjacent free blocks. This can lead to serious memory fragmentation over time.

- heap_4: (Recommended) Combines (coalesces) adjacent free blocks of memory to prevent fragmentation. It uses a first-fit algorithm and is highly reliable for dynamic applications.

- heap_5: Similar to heap_4, but allows the heap to span across multiple physically separated memory banks (e.g., internal SRAM and external SDRAM).

### The Danger of Stack Overflow

Because microcontrollers have limited RAM (the STM32F446RE has 128 KB of SRAM), defining too many local arrays or nesting too many function structures inside a task can cause its stack pointer to step out of bounds. This is a Stack Overflow. It silently overwrites neighboring variables, kernel structures, or other task stacks, causing unpredictable system crashes.

### Stack Overflow Detection Hooks

FreeRTOS can actively monitor task stacks using two methods configured via configCHECK_FOR_STACK_OVERFLOW:

- Method 1: The kernel checks if the current stack pointer has exceeded the valid stack limit during a context switch. (Fast, but might miss an overflow if it happens and resolves between switches).

- Method 2: The kernel populates the last 16 bytes of a task's stack with a specific dummy pattern (e.g., 0xA5A5A5A5) when the task is created. During a context switch, it checks if these bytes have been overwritten. If they are changed, the kernel raises a panic by calling the Application Stack Overflow Hook Function.

## 4. Architectural Configuration in STM32CubeMX

To configure and capture memory vulnerabilities safely:

- Enable Memory Hook Intercepts: Under Middleware > FREERTOS > Config parameters, scroll down to find configCHECK_FOR_STACK_OVERFLOW. Set this parameter to Option 2 (Method 2).

- Configure Total Heap Size: In the same parameters list, check TOTAL_HEAP_SIZE. For the STM32F446RE, this is typically allocated to around 15360 Bytes or higher depending on allocation requirements.

- Task Setup: Create a testing thread with a modest stack allocation space (e.g., 128 words / 512 bytes).

## 5. Firmware Implementation Structure

The following structural template demonstrates how to declare the application memory hook and intentionally trigger a stack overflow using an uncontrolled recursive call profile to test system resilience.

C

/* Includes ------------------------------------------------------------------*/

#include "main.h"

#include "cmsis_os.h"

#include <stdio.h>

/* Private variables ---------------------------------------------------------*/

// Thread Handle for a volatile testing task

osThreadId OverflowTestTaskHandle;

/* UART handle for fault monitoring */

extern UART_HandleTypeDef huart2;

/* Private function prototypes -----------------------------------------------*/

void StartOverflowTestTask(void const * argument);

void InduceStackOverflow(uint32_t iteration);

int main(void)

{

/* MCU Configuration -------------------------------------------------------*/

HAL_Init();

SystemClock_Config();

MX_GPIO_Init();

MX_USART2_UART_Init();

char msg_init[] = "System Initialized. Starting Stack Monitoring Experiment...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_init, sizeof(msg_init)-1, 100);

/* --- Create a Task with a small stack size (128 words = 512 bytes) --- */

osThreadDef(OverflowTestTask, StartOverflowTestTask, osPriorityNormal, 0, 128);

OverflowTestTaskHandle = osThreadCreate(osThread(OverflowTestTask), NULL);

/* Start RTOS Scheduler */

osKernelStart();

while (1)

{

// Kernel controlled loop

}

}

/*------------------------- Application Memory Hook --------------------------*/

/**

* @brief  FreeRTOS Stack Overflow Application Hook.

* This function overrides a weak kernel signature and is automatically called

* by the scheduler if a task breaks its allocated stack bounds.

*/

void vApplicationStackOverflowHook(xTaskHandle xTask, signed char *pcTaskName)

{

char panic_buf[80];

/* CRITICAL ALERT: The stack is corrupted. Do not create local variables or

call deep functions here. Immediately report the issue and halt. */

int len = snprintf(panic_buf, sizeof(panic_buf),

"\r\n!!! [KERNEL PANIC] Stack Overflow detected in Task: %s !!!\r\n",

pcTaskName);

HAL_UART_Transmit(&huart2, (uint8_t*)panic_buf, len, HAL_MAX_DELAY);

// Turn on a fault indicator LED or enter an infinite safety loop

while(1)

{

HAL_GPIO_TogglePin(GPIOA, GPIO_PIN_5); // Toggle Onboard Green LED rapidly

HAL_Delay(100);

}

}

/*------------------------- Task Implementations -----------------------------*/

/**

* @brief  Recursive function designed to force local stack context growth.

*/

void InduceStackOverflow(uint32_t iteration)

{

// Allocate a dummy buffer on the local stack to absorb space rapidly

uint8_t large_local_buffer[64];

large_local_buffer[0] = (uint8_t)iteration;

// Print progress via non-blocking call

char trace_msg[40];

int len = snprintf(trace_msg, sizeof(trace_msg), "Recursion Depth: %lu\r\n", iteration);

HAL_UART_Transmit(&huart2, (uint8_t*)trace_msg, len, 100);

// Uncontrolled recursive call - creates infinite stack frames until overflow occurs

InduceStackOverflow(iteration + 1);

}

/**

* @brief  Overflow Test Task: Triggers the recursive failure vector.

*/

void StartOverflowTestTask(void const * argument)

{

// Give the system a brief moment to settle

osDelay(1000);

char msg_start[] = "Target task running. Inducing memory crash...\r\n";

HAL_UART_Transmit(&huart2, (uint8_t*)msg_start, sizeof(msg_start)-1, 100);

// Call the stack-consuming routine

InduceStackOverflow(1);

for(;;)

{

osDelay(1);

}

}

## 6. Procedural Steps for Lab Execution

- Load Target Project: Open your configured workspace project framework in STM32CubeIDE.

- Turn on the Diagnostic Hooks: Open the .ioc configuration canvas. Navigate to Middleware > FREERTOS > Config parameters. Find configCHECK_FOR_STACK_OVERFLOW and switch its value to Option 2.

- Configure Memory Footprint: Ensure TOTAL_HEAP_SIZE is adequately set to allocate tasks cleanly.

- Generate Code Scaffolding: Press Ctrl + S to save changes and update your system files.

- Implement Hook Logic: Open main.c. Scroll down to the user code sections and define void vApplicationStackOverflowHook(xTaskHandle xTask, signed char *pcTaskName).

- Add the Failure Vector: Add the recursive test code block (InduceStackOverflow) inside your testing task function to trigger the crash conditions.

- Compile and Run: Build the code, flash it onto the board, and open your serial monitoring tool to observe the output when the kernel catches the overflow error.

## 7. Observations & Verification

- Graceful Failure Capture: On running the application, the terminal prints a few increments of Recursion Depth. As soon as the recursion consumes more than 512 bytes, the serial window instantly outputs the panic log: !!! [KERNEL PANIC] Stack Overflow detected in Task: OverflowTestTask !!!.

- System Hardening: Instead of silently freezing or behaving erratically, the system safely identifies the memory fault and drops into the infinite diagnostic safety loop (vApplicationStackOverflowHook), demonstrating a production-grade approach to debugging memory issues in embedded software.